STEP 1 — Define the Inputs for Your Lifecycle Emissions Calculator

We’re building this like a real consultant would: clean, structured, and scalable.

Below are the exact inputs your tool will need. These will become the first rows in your Excel/Sheets model.

A. Vehicle Inputs

These describe the car itself.

- Vehicle type: ICE or BEV

- Battery size (kWh): for BEV

- Electricity consumption (kWh per 100 miles): BEV

- Fuel economy (miles per gallon): ICE

B. Regional Inputs

These describe the region’s energy profile.

- Grid carbon intensity (kg CO₂ per kWh): Minnesota

- Fuel carbon intensity (kg CO₂ per gallon): U.S. standard

- Charging behavior assumption: home charging vs mixed

C. Lifecycle Inputs

These describe manufacturing emissions.

- ICE manufacturing emissions (kg CO₂ per vehicle)

- BEV manufacturing emissions (kg CO₂ per vehicle)

- Battery manufacturing emissions (kg CO₂ per kWh)

D. Usage Inputs

These describe how the car is driven.

- Lifetime mileage (miles)

- Annual mileage (optional)

These inputs are universal — meaning you can swap BMW for Toyota, Tesla, Honda, BYD, anything.

🔥 STEP 2 — Define the Outputs

Your tool will calculate:

- Upstream emissions (ICE vs BEV)

- Downstream emissions (ICE vs BEV)

- Total lifecycle emissions (ICE vs BEV)

- Difference (ICE – BEV)

- ICE Downstream emissions per mile

- BEV Downstream emissions per mile

- Breakeven mileage (This is the heart of lifecycle modeling).

Optional refinements (for future versions)

These are things you can add later if you want to make a “Version 2” or “Pro” version:

- Charging losses (5–15%)

- Seasonal electricity variation

- Battery degradation

- Regional grid mix changes over time

- Manufacturing location differences

- Recycling credits

- Vehicle weight differences

🔥 STEP 3 — Build the Excel Structure

Here’s the exact layout you’ll create in Excel or Google Sheets.

SECTION 1 — Inputs (User-Editable)

| Input | Value | Notes |
| --- | --- | --- |
| Vehicle type | ICE / BEV | Dropdown |
| Battery size (kWh) | 80 | Example for BMW i4 |
| Electricity consumption (kWh/100 miles) | 30 | Typical BEV |
| Fuel economy (mpg) | 30 | Typical ICE sedan |
| Grid carbon intensity (kg CO₂/kWh) | Minnesota value | We’ll fill this |
| Fuel carbon intensity (kg CO₂/gallon) | 8.89 | U.S. EPA |
| ICE manufacturing emissions (kg CO₂) | 6000 | Industry average |
| BEV manufacturing emissions (kg CO₂) | 8000 | Industry average |
| Battery manufacturing emissions (kg CO₂/kWh) | 80 | Mid-range estimate |
| Lifetime mileage (miles) | 150,000 | User input |

This is your control panel.

🔥 STEP 4 — Formulas (We’ll build these next)

We will calculate:

Upstream Emissions

- ICE upstream = ICE manufacturing

- BEV upstream = BEV manufacturing + (battery size × battery manufacturing factor)

Downstream Emissions

- ICE downstream = (lifetime miles ÷ mpg) × fuel carbon intensity

- BEV downstream = (lifetime miles ÷ 100) × electricity consumption × grid carbon intensity

Total Lifecycle

- ICE total = upstream + downstream

- BEV total = upstream + downstream

Breakeven Mileage

We’ll derive a formula that solves for the mileage where:

BEV total(miles)=ICE total(miles)

This is where the BEV becomes cleaner.

A. Vehicle Inputs

BMW X3 (ICE)

- Fuel economy: 25 mpg (combined)

- Manufacturing emissions: 6,000 kg CO₂ (industry average)

BMW iX3 (BEV)

- Battery size: 80 kWh

- Electricity consumption: 30 kWh per 100 miles

- Manufacturing emissions (vehicle): 8,000 kg CO₂

- Battery manufacturing emissions: 80 kg CO₂ per kWh

So battery emissions = 80 kWh×80 kg CO₂/kWh=6,400 kg CO₂

B. Regional Inputs — Minnesota

Minnesota grid carbon intensity (average): 0.40 kg CO₂ per kWh

Fuel carbon intensity (U.S. EPA): 8.89 kg CO₂ per gallon

C. Usage Inputs

Let’s start with a standard assumption: Lifetime mileage: 150,000 miles

(You can change this anytime.)

🧩 STEP 6 — Build the Excel Layout (Your First Real Skill)

Open Excel or Google Sheets and create this exact structure:

SECTION 1 — INPUTS

| Input | Value | Notes |
| --- | --- | --- |
| ICE fuel economy (mpg) | 25 | BMW X3 |
| BEV battery size (kWh) | 80 | BMW iX3 |
| BEV electricity use (kWh/100 miles) | 30 | BMW iX3 |
| Grid carbon intensity (kg CO₂/kWh) | 0.40 | Minnesota |
| Fuel carbon intensity (kg CO₂/gallon) | 8.89 | U.S. EPA |
| ICE manufacturing emissions (kg CO₂) | 6000 | Industry avg |
| BEV manufacturing emissions (kg CO₂) | 8000 | Industry avg |
| Battery manufacturing factor (kg CO₂/kWh) | 80 | Industry avg |
| Lifetime mileage (miles) | 150000 | User input |

🧮 STEP 7 — Add the Formulas (I’ll give you each one next)

We will calculate:

Upstream Emissions

- ICE upstream = ICE manufacturing

- BEV upstream = BEV manufacturing + (battery size × battery factor)

Downstream Emissions

- ICE downstream = (lifetime miles ÷ mpg) × fuel carbon intensity

- BEV downstream = (lifetime miles ÷ 100) × electricity use × grid carbon intensity

Total Lifecycle

- ICE total = upstream + downstream

- BEV total = upstream + downstream

Breakeven Mileage

We’ll derive the formula together — step by step.